User’s Manual

Models  VFD220H

Vacuum Fluorescent Customer Display

INTERFACE SPECIFICATIONS

1. Communication

The interface specification are based on EIA RS232C baud rate 9600 bps, 8 data bits, no parity, 1 stop bits.

2. RS232C Connector

Connector type: D-sub 9 pin (Female)

| Pin number | Signal name | signal dirction DISPLAY-HOST |
| --- | --- | --- |
| 1 | CD | ----- |
| 2 | RXD | ----- |
| 3 | TXD | <----- |
| 4 | DTR | ----- |

| 5 | GND | <---> |
| --- | --- | --- |
| 6 | DSR | ----- |
| 7 | RTS | ----- |
| 8 | CTS | ----- |
| 9 | RI | ----- |

3. Power connector

Connector type: USB POWER (DC5V)

)

General  commands  instruction set list:

- STX  B  n

Name：select baud rate

ACSII：STX  B  n

Decimal：002  066  n

Hex：02H  42H  n

Description：When the power is turned on, the default value of n is 0(set to 9600)

| ASCII n | Decimal n | Hex n | Baud rate |
| --- | --- | --- | --- |
| 0 | 48 | 30H | 9600 |
| 1 | 49 | 31H | 4800 |
| 2 | 50 | 32H | 2400 |
| 3 | 51 | 33H | 1200 |
| 4 | 52 | 34H | 600 |
| 5 | 53 | 35H | 300 |

- STX  C  n

Name：select command set

ACSII：STX  C  n

Decimal：002  067  n

Hex：02H  43H  n

Description：When the power is turned on, the default value of n is 0(set to CD5200/20)

| ASCII n | Decimal n | Hex n | Command set |
| --- | --- | --- | --- |
| 0 | 48 | 30H | CD5200/20 |
| 1 | 49 | 31H | ESC/POS |

- STX  MD5  n

Name：select international fonts set

ACSII ：STX  MD5  n

Decimal： 002  005  n       48<=n<=60

Hex：02H  05H  n     30H<=n<=3CH

Description：select international fonts and stored in the internal EEPROM, the next power will continue to use this setting. When the power is turned on, the default value of n is A(set to U.S.A.)

| ASCII n | Decimal n | Hex n | Country |
| --- | --- | --- | --- |
| 0 | 48 | 30H | U.S.A |
| 1 | 49 | 31H | FRANCE |
| 2 | 50 | 32H | GERMANY |
| 3 | 51 | 33H | U.K |
| 4 | 52 | 34H | DENMARKI |
| 5 | 53 | 35H | SWEDEN |
| 6 | 54 | 36H | ITALY |
| 7 | 55 | 37H | SPAIN |
| 8 | 56 | 38H | JAPAN |
| 9 | 57 | 39H | NORWAY |
| A | 58 | 3AH | DENMARKII |
| B | 59 | 3BH | SLAVONIC |
| C | 60 | 3CH | RUSSIA |

select international fonts  the character differences, please refer to the following:

- STX  MD6  n

Name：Select the Extended Character Font Table

ACSII ：STX  MD6  n

Decimal：002  006   n      48<=n<=56

Hex：：02H   06H   n     30H<=n<=38H

Description：Select extended character font table and stored in the internal EEPROM, the next power will continue to use this setting.Use the font stored in 80H ... FFH.

| ASCII n | Decimal n | Hex n | Extended Character Font Table(80H…FFH) |
| --- | --- | --- | --- |
| 0 | 48 | 30H | U.S.A. and Standard Europe |
| 1 | 49 | 31H | Katakana for Japan |
| 2 | 50 | 32H | Multilingual |
| 3 | 51 | 33H | Portuguese |
| 4 | 52 | 34H | Canadian French |
| 5 | 53 | 35H | Nordic |
| 6 | 54 | 36H | SLAVONIC |
| 7 | 55 | 37H | RUSSIA |
| 8 | 56 | 38H | Turkish |

CD 5200/20

commands instruction set list

- ESC  DC1

Name：Select overwrite mode

ACSII：ESC  DC1

Decimal：27  17

Hex：1B  11

Description：In overwriting mode, entering a character code moves to the left end of the lower line when the cursor is at the right end of the upper line, and to the left end of the upper line when the cursor is at the right end of the lower line. When the power is turned on, this mode is selected defaultly.

- ESC  DC2

Name：Select vertical scroll mode

ACSII：ESC  DC2

Decimal：27  18

Hex：1B  12

Description：In vertical scroll mode, entering a character code moves the cursor to the left end of the lower line when the cursor ia at the right end of the upper line, scrolls the characters displayed on the lower line, and clears the lower line when the cursor is at the right end of the lower line. At this time the cursor is moved to the left end of the lower line.

- ESC  DC3

Name：Select horizontal scroll mode

ACSII：ESC  DC3

Decimal：27  19

Hex：1B  13

Description：In horizontal scroll mode, entering a character code scrolls all displayed characters one character to the left, then displays the new character at the right end.

- ESC  Q  A  d1  d2  d3  ……  dn  CR

Name：set the string display mode, write string to upper line

ACSII：ESC  Q  A  d1  d2  d3  ……  dn  CR

Decimal：27  81  65  d1  d2  d3  ……  dn  13

Hex：1B  51  41  d1  d2  d3  ……  dn  0D

Description：write string (d1  d2  d3  ……  dn) to upper line. 20H<=dn<=FFH, 1<= n<=20.

- ESC  Q  B  d1  d2  d3  ……  dn  CR

Name：set the string display mode, write string to lower line

ACSII：ESC  Q  B  d1  d2  d3  ……  dn  CR

Decimal：27  81  66  d1  d2  d3  ……  dn  13

Hex：1B  51  42  d1  d2  d3  ……  dn  0D

Description：write string (d1  d2  d3  ……  dn) to lower line. 20H<=dn<=FFH, 1<= n<=20.

- ESC  Q  D  d1  d2  d3  ……  dn  CR

Name：upper line message scroll continuously

ACSII：ESC  Q  D  d1  d2  d3  ……  dn  CR

Decimal：27  81  68  d1  d2  d3  ……  dn  13

Hex：1B  51  44  d1  d2  d3  ……  dn   0D

Description：upper line message (d1  d2  d3  ……  dn) scroll continuously. 20H<=dn<=FFH, 1<= n<=20.

- ESC  [  D

Name：move the cursor one character position to the left

ACSII：ESC  [  D

Decimal：27  91  68

Hex：1B  5B	  44

Description：When the cursor is at the left end of a line, the operation of this command depends on the display mode.

1) Overwrite mode : When the cursor is at the end of the lower line, it is moved to the right end of the upper line. When it is at the end of the upper line, it is moved to the right end of the lower line.

2) Vertical scroll mode : When the cursor is at the end of the lower line, it is moved to the right end of the upper line. When it is at the end of the upper line, the display on the upper line is scrolled to the lower line and the upper line is cleared. At this time the cursor is moved to the right end of the upper line.

3) Horizontal scroll mode : All characters on the current line are scrolled on character to the right. The cursor is not moved, but the character area at the left end is cleared.

- BS

Name：move the cursor one character position to the left

ACSII：BS

Decimal：8

Hex：08

Description：When the cursor is at the left end of a line, the operation of this command depends on the display mode.

1) Overwrite mode : When the cursor is at the end of the lower line, it is moved to the right end of the upper line. When it is at the end of the upper line, it is moved to the right end of the lower line.

2) Vertical scroll mode : When the cursor is at the end of the lower line, it is moved to the right end of the upper line. When it is at the end of the upper line, the display on the upper line is scrolled to the lower line and the upper line is cleared. At this time the cursor is moved to the right end of the upper line.

3) Horizontal scroll mode : All characters on the current line are scrolled on character to the right. The cursor is not moved, but the character area at the left end is cleared.

- ESC  [  C

Name：move the cursor one character position to the right

ACSII：ESC  [  C

Decimal：27  91  67

Hex：1B  5B  43

Description：When the cursor is at the right end of a line, the operation of this command depends on the display mode.

1) Overwrite mode : When the cursor is at the right end of the upper line, it is moved to the left end of the lower line. When it is at the right end of the lower line, it is moved to the left end of the upper line.

2) Vertical scroll mode : When the cursor is at the right end of the upper line, it is moved to the left end of the lower line. When it is at the right end of the lower line, the display on the lower line is scrolled to the upper line and the lower line is cleared.At this time, the cursor is moved to the left end of the lower line.

3)Horizontal scroll line : All characters on the current line are scrolled one character to the left.The cursor is not moved, but the character area at the right end is cleared.

- HT

Name：move the cursor one character position to the right

ACSII：HT

Decimal：9

Hex：09

Description：When the cursor is at the right end of a line, the operation of this command depends on the display mode.

1) Overwrite mode : When the cursor is at the right end of the upper line, it is moved to the left end of the lower line. When it is at the right end of the lower line, it is moved to the left end of the upper line.

2) Vertical scroll mode : When the cursor is at the right end of the upper line, it is moved to the left end of the lower line. When it is at the right end of the lower line, the display on the lower line is scrolled to the upper line and the lower line is cleared.At this time, the cursor is moved to the left end of the lower line.

3)Horizontal scroll line : All characters on the current line are scrolled one character to the left.The cursor is not moved, but the character area at the right end is cleared.

- ESC  [  A

Name：move the cursor up one line

ACSII：ESC  [  A

Decimal：27  91  65

Hex：1B  5B  41

Description：When the cursor is on the upper line, the operation of this command depends on the display mode,as follows:

1)Overwrite mode :The cursor is moved to the same column on the lower line.

2)Vertical scroll mode :The characters displayed on the upper line are scrolled to the lower line and the upper line iscleared. The cursor remains at the same position.

3)Horizontal scroll mode :The cursor is not moved.

- ESC  [  B

Name：move the cursor down one line

ACSII：ESC  [  B

Decimal：27  91  66

Hex：1B  5B  42

Description：When the cursor is on the lower line, the operation of this command depends on the display mode,as follows:

1)Overwrite mode :The cursor is moved to the same column on the upper line.

2)Vertical scroll mode :The characters displayed in the lower line are scrolled to the upper line and the lower line is cleared. The cursor remains at the same position.

3)Horizontal scroll mode :The cursor is not moved.

`

- LF

Name：move the cursor down one line

ACSII：LF

Decimal：10

Hex：0A

Description：When the cursor is on the lower line, the operation of this command depends on the display mode,as follows:

1)Overwrite mode :The cursor is moved to the same column on the upper line.

2)Vertical scroll mode :The characters displayed in the lower line are scrolled to the upper line and the lower line is cleared. The cursor remains at the same position.

3)Horizontal scroll mode :The cursor is not moved.

- ESC  [  H

Name：move cursor to home position

ACSII：ESC  [  H

Decimal：27  91  72

Hex：1B  5B  48

Description：Moves the cursor to the left-end position on the upper line (home position).Home position indicates the first column of the upper line.

- HOM

Name：move the cursor to the left-end position on the upper line (home position)

ACSII：HOM

Decimal：11

Hex：0B

- ESC  [  K

Name：move the cursor to the 20th column of the lower line

ACSII：ESC  [  K

Decimal：27  91  75

Hex：1B  5B  4B

- ESC  [  L

Name：move the cursor to left-most position on the current line

ACSII：ESC  [  L

Decimal：27  91  76

Hex：1B  5B  4C

- CR

Name：move the cursor to left-most position on the current line

ACSII：CR

Decimal：13

Hex：0D

- ESC  [  R

Name：move the cursor to the right-end position on the current line

ACSII：ESC  [  R

Decimal：27  91  82

Hex：1B  5B  52

- ESC  l  x  y

Name：move cursor to specified position

ACSII：ESC  l  x  y

Decimal：27  108  x  y

Hex：1B  6C  x  y

Description：moves the cursor to the xth column on the yth line.1<=x<=20,y=1,2

- ESC  @

Name：initialize display

ACSII：ESC  @

Decimal：27  64

Hex：1B  40

Description：The software settings are reset to their power-on values. After initializing the display, the display screen is cleared and the cursor moves to the home position.

- CLR

Name：clear display screen

ACSII：CLR

Decimal：12

Hex：0C

Description：Clear all the displayed characters.After this command is executed, the cursor moves to the home position.

- CAN

Name：clear cursor line, and clear string mode

ACSII：CAN

Decimal：24

Hex：18

Description：Clears the line containing the cursor.After this command is executed, the cursor moves to the left-end position on the current line.

- ESC  *  n

Name：brightness adjustment

ACSII：ESC  *  n

Decimal：27  42  n

Hex：1B  2A  n

Description：Sets the brightness of the vacuum fluorescent character display tube.n selects the percentage of brightness as follows:

n = 1, Brightness Level = 40 %

n = 2, Brightness Level = 60 %

n = 3, Brightness Level = 80 %

n = 4, Brightness Level =100 %

When the power is turned on, n =4 is selected defaultly.

- ESC  _  n

Name：set cursor ON/OFF

ACSII：	ESC  _  n

Decimal：27  95  n

Hex：1B  5F  n

Description：Turns the cursor on or off.

When n =0, the cursor is turned off.

When n =1, the cursor is turned on.

- ESC  f  n

Name：select international fonts set

ACSII：ESC  f  n

Decimal：27  102  n

Hex：1B  66  n

Description：When the power is turned on, the default value of n is A(set to U.S.A.)

| ASCII n | Decimal n | Hex n | Country |
| --- | --- | --- | --- |
| A | 65 | 41H | U.S.A |
| D | 68 | 44H | DENMARKI |
| E | 69 | 45H | DENMARKII |
| F | 70 | 46H | FRANCE |
| G | 71 | 47H | GERMANY |
| I | 73 | 49H | ITALY |
| J | 74 | 4AH | JAPAN. |
| L | 76 | 4CH | SLAVONIC |
| N | 78 | 4EH | NORMAY |
| R | 82 | 52H | RUSSIA |
| S | 83 | 53H | SPAIN |
| U | 85 | 55H | U.K. |
| W | 87 | 57H | SWEDEN |

- ESC  c  n

Name：select fonts ,ASCII code or JIS code

ACSII：	ESC  c  n

Decimal：27  99  n

Hex：1B  63  n

Description：When the power is turned on, the default value of n is A(set to U.S.A. and Standard Europe)

| ASCII n | Decimal n | Hex n | character code table(80H…FFH) |
| --- | --- | --- | --- |
| A | 64 | 41H | U.S.A. and Standard Europe |
| J | 74 | 4AH | Katakana for Japan |
| L | 76 | 4CH | SLAVONIC |
| R | 82 | 52H | RUSSIA |

EPSON   ESC/POS

commands instruction set list

- US  MD1

Name：Select overwrite mode

ACSII：US  MD1

Decimal：31  1

Hex：1F  01

Description：In overwriting mode, entering a character code moves to the left end of the lower line when the cursor is at the right end of the upper line, and to the left end of the upper line when the cursor is at the right end of the lower line. When the power is turned on, this mode is selected defaultly.

- US  MD2

Name：Select vertical scroll mode

ACSII：US  MD2

Decimal：31  2

Hex：1F  02

Description：In vertical scroll mode, entering a character code moves the cursor to the left end of the lower line when the cursor ia at the right end of the upper line, scrolls the characters displayed on the lower line, and clears the lower line when the cursor is at the right end of the lower line. At this time the cursor is moved to the left end of the lower line.

- US  MD3

Name：Select horizontal scroll mode

ACSII：US  MD3

Decimal：31  3

Hex：1F  03

Description：In horizontal scroll mode, entering a character code scrolls all displayed characters one character to the left, then displays the new character at the right end.

- BS

Name：move the cursor one character position to the left

ACSII：BS

Decimal：8

Hex：08

Description：When the cursor is at the left end of a line, the operation of this command depends on the display mode.

1) Overwrite mode : When the cursor is at the end of the lower line, it is moved to the right end of the upper line. When it is at the end of the upper line, it is moved to the right end of the lower line.

2) Vertical scroll mode : When the cursor is at the end of the lower line, it is moved to the right end of the upper line. When it is at the end of the upper line, the display on the upper line is scrolled to the lower line and the upper line is cleared. At this time the cursor is moved to the right end of the upper line.

3) Horizontal scroll mode : All characters on the current line are scrolled on character to the right. The cursor is not moved, but the character area at the left end is cleared.

- HT

Name：move the cursor one character position to the right

ACSII：HT

Decimal：9

Hex：09

Description：When the cursor is at the right end of a line, the operation of this command depends on the display mode.

1) Overwrite mode : When the cursor is at the right end of the upper line, it is moved to the left end of the lower line. When it is at the right end of the lower line, it is moved to the left end of the upper line.

2) Vertical scroll mode : When the cursor is at the right end of the upper line, it is moved to the left end of the lower line. When it is at the right end of the lower line, the display on the lower line is scrolled to the upper line and the lower line is cleared.At this time, the cursor is moved to the left end of the lower line.

3)Horizontal scroll line : All characters on the current line are scrolled one character to the left.The cursor is not moved, but the character area at the right end is cleared.

- US  LF

Name：move the cursor up one line

ACSII：US  LF

Decimal：31  10

Hex：1F  0A

Description：When the cursor is on the upper line, the operation of this command depends on the display mode,as follows:

1)Overwrite mode :The cursor is moved to the same column on the lower line.

2)Vertical scroll mode :The characters displayed on the upper line are scrolled to the lower line and the upper line iscleared. The cursor remains at the same position.

3)Horizontal scroll mode :The cursor is not moved.

- LF

Name：move the cursor down one line

ACSII：LF

Decimal：10

Hex：0A

Description：When the cursor is on the lower line, the operation of this command depends on the display mode,as follows:

1)Overwrite mode :The cursor is moved to the same column on the upper line.

2)Vertical scroll mode :The characters displayed in the lower line are scrolled to the upper line and the lower line is cleared. The cursor remains at the same position.

3)Horizontal scroll mode :The cursor is not moved.

- CR

Name：move the cursor to left-most position on the current line

ACSII：CR

Decimal：13

Hex：0D

- US  CR

Name：move the cursor to the right-end position on the current line

ACSII：US  CR

Decimal：31  13

Hex：1F  0D

- HOM

Name：move the cursor to the left-end position on the upper line (home position)

ACSII：HOM

Decimal：11

Hex：0B

- US  B

Name：move the cursor to the 20th column of the lower line

ACSII：US  B

Decimal：31  66

Hex：1F  42

- US  $  x  y

Name：move cursor to specified position

ACSII：US  $  x  y

Decimal：31  36  x  y

Hex：1F  24  x  y

Description：moves the cursor to the xth column on the yth line.1<=x<=20,y=1,2

- ESC  @

Name：initialize display

ACSII：ESC  @

Decimal：27  64

Hex：1B  40

Description：The software settings are reset to their power-on values. After initializing the display, the display screen is cleared and the cursor moves to the home position.

- CLR

Name：clear display screen

ACSII：CLR

Decimal：12

Hex：0C

Description：Clear all the displayed characters.After this command is executed, the cursor moves to the home position.

- CAN

Name：clear cursor line, and clear string mode

ACSII：CAN

Decimal：24

Hex：18

Description：Clears the line containing the cursor.After this command is executed, the cursor moves to the left-end position on the current line.

- US  C  n

Name：set cursor ON/OFF

ACSII：	US  C  n

Decimal：31  67  n

Hex：1F  43  n

Description：Turns the cursor on or off.

When n =0, the cursor is turned off.

When n =1, the cursor is turned on.

- US  .  n

Name：display char ‘n’ and  ‘.’

ACSII：US  .  n

Decimal：31  46  n

Hex：1F  2E  n

Description：20H<=n<=FFH

- US  ,  n

Name：display char ‘n’ and  ‘,’

ACSII：US  ,  n

Decimal：[031][044]n

Hex：[1FH][2CH]n

Description：20H<=n<=FFH

- US  ;  n

Name：display char ‘n’ and  ‘;’

ACSII：US  ;  n

Decimal：31  59  n

Hex：1F  3B  n

Description：20H<=n<=FFH

- US  X  n

Name：brightness adjustment

ACSII：US  X  n

Decimal：31  42  n

Hex：1F  2A  n

Description：Sets the brightness of the vacuum fluorescent character display tube.n selects the percentage of brightness as follows:

n = 1, Brightness Level = 40 %

n = 2, Brightness Level = 60 %

n = 3, Brightness Level = 80 %

n = 4, Brightness Level =100 %

When the power is turned on, n =4 is selected defaultly.

- US  E  n

Name：Sets or cancels the blink interval of the display screen

ACSII：US  E  n

Decimal：31  69  n

Hex：1F  45  n

Description：n specifies the blink interval. When n = 0, the display is kept on(cancels blinking).

When n = FF Hex (255 decimal), the display is turned off but the contents of the display are maintained.

When the power is turned on, n = 0 is selected defaultly.

- ESC  R  n

Name：select international fonts set

ACSII：ESC  R  n

Decimal：[027][082]n       32<=n<=44

Hex：[1BH][52H]n     00H<=n<=0CH

Description：When the power is turned on, the default value of n is SPACE(set to U.S.A.)

| Decimal n | Hex n | Country |
| --- | --- | --- |
| 0 | 00H | U.S.A |
| 1 | 01H | FRANCE |
| 2 | 02H | GERMANY |
| 3 | 03H | U.K. |
| 4 | 04H | DENMARKI |
| 5 | 05H | SWEDEN |
| 6 | 06H | ITALY |
| 7 | 07H | SPAIN |
| 8 | 08H | JAPAN. |
| 9 | 09H | NORMAY |
| 10 | 0AH | DENMARKII |
| 11 | 0BH | SLAVONIC |
| 12 | 0CH | RUSSIA |

- ESC  t  n

Name：select fonts ,ASCII code or JIS code

ACSII：	ESC  t  n

Decimal：[027][116] n

Hex：[1BH][74H] n

Description：When the power is turned on, the default value of n is 0(set to U.S.A. and Standard Europe)

| Decimal n | Hex n | character code table(80H…FFH) |
| --- | --- | --- |
| 0 | 00H | U.S.A. and Standard Europe |
| 1 | 01H | Katakana for Japan |
| 2 | 02H | Multilingual |
| 3 | 03H | Portuguese |
| 4 | 04H | Canadian French |
| 5 | 05H | Nordic |
| 6 | 06H | SLAVONIC |
| 7 | 07H | RUSSIA |